NTW275: Standard Operating Procedure

Security and NAT Policies

Author: Jose Ramirez

Date: 3/28/2020

SOP Number: Sec-1

Related SOP’s: N/A

PURPOSE:

The firewall needs NAT policies to be able to translate internal IP addresses to external. The security policies are used to determine what applications, services, or URL’s are safe to access.

REQUIREMENTS:

The following equipment is needed to complete the procedure:

- Palo Alto Firewall

- Windows 10 machine with Google Chrome installed

PROCEDURE:

Logging Into The Palo Alto Firewall:

Open Google Chrome and navigate to the firewall’s log in page. This will be the IP address assigned to the firewall’s interface. Make sure you navigate to the page through HTTPS.

Figure 1.1 - Palo Alto Log-In Screen.

For example, the firewall is located at 192.168.1.254, therefore the site would be https://192.168.1.254 . Log into the web interface. (fig. 1.1)

Loading Pre-Existing Configurations On The Firewall:

If there are any pre-existing configurations they will be stored under the Device tab. On the selection panel to the left select Setup and select the Operations tab.

Under this tab click on Load named configuration snapshot. (fig. 2.1)

Figure 2.1 - Devices > Setup > Operations > Load named configuration snapshot

In the window that appears select the configuration that needs to be loaded from the drop-down menu. Enter a decryption key if needed. Click OK when you are finished with the selection. (fig. 2.2)

Figure 2.2 - Select the configuration and click on OK

A Loading Configuration window will appear stating the configuration is being loaded. Click Close.

Before the changes are made, a commit must be made. Click on the Commit link, located on the top right of the dashboard. (fig. 2.3)

Figure 2.3 - Click on the Commit link

This will open the Commit window. Review the changes if they are displayed. Click Commit when finished. The commit will be processed, wait until it is complete. (fig. 2.4)

A Commit Status window will open stating the commits status and result. Click Close. (fig. 2.5)

Note: Commits are a common process done when saving changes. This is done constantly throughout configurations. Refer to this section if needed.

Figure 2.4 - Click Commit

Figure 2.5 - Close out of the window

Creating Tags:

Tags are color coded labels used to identify groups and policies. They can be applied to many other objects during configurations as well.

Head over to the Objects tab and click on Tags in the left panel.

Click on Add in the bottom left side of the window.

Figure 3.1 - Devices > Tags > Add

Creating tags is simple, set the Name, the Color, and the Comments for the tag.

Click OK when finished.

Create tags as needed by referring to this section.

Figure 3.2 - Set Name, Color, Comments

Commit the changes.

Creating NAT Policies:

NAT is used to translate private IP addresses to public IP addresses. This is used to access resources outside of the network. Refer to this section when creating NAT policies.

Head over to the Policies tab and click on NAT in the left panel.

Click on Add to create a new policy.

Figure 4.1 - Policies > NAT > Add

This will open a new window called NAT Policy Rule.

Set the Name, add Tags and Group Rules By Tag, set NAT Type and add a Audit Comment if needed.

Figure 4.2 - Set Name, Tags, and NAT Type.

Move over to the Original Packet tab in the window.

Click Add to add a new Source Zone. For this example, I have chosen the inside zone.

Set Destination Zone to the zone the traffic will be headed to, set Destination Interface to the interface handling the destination, and set Service to the service being used. For this example, the Service is Any service.

For this example, I have set Source Address and Destination Address to Any but append as needed.

Figure 4.3 - Set Source Zone, Destination Zone, Destination Interface, Service, Source Address, and Destination Address

Move over to the Translated Packet tab in the same window.

For this example, I have set the Translation Type to Dynamic IP And Port, Address Type to Interface Address, Interface to ethernet1/1, and  IP Address to 203.0.113.20/24.

Append as needed and click OK when finished.

Figure 4.4 - Set Translation Type, Address Type, Interface, and IP Address

Commit the changes.

Creating Security Policies:

Security policies are used to enforce traffic rules and will allow you to block, track, allow, etc. traffic into your network. This can be set to certain applications, services, users, or user groups. Refer to this section when creating Security Policies.

Head over to the Policies tab and click on Security in the left panel.

Click Add on the bottom left to create a new security policy.

Figure 5.1 - Policies > Security > Add

A new window will appear called Security Policy Rule

Set the Name, set Rule Type, add any Tags that are needed, set the Group Rule By Tag to a tag previously set, and add a Audit Comment if needed.

Figure 5.2 - Set Name, Rule Type, Tags, Group Rule By Tag, and add a Audit Comment.

Move to the Source tab in the same window.

Click on Add in the Source Zone section and add in a source to the rule. For this example, the Source Zone would be inside. Set the Source Address to Any.

Figure 5.3 - Set Source Zone or Add one, and Source Address

Move to the Destination tab in the window.

Set the Destination Zone and the Destination Address. For this example, the Destination Zone outside was added, and the Destination Address was set to Any.

Figure 5.4 - Set or Add Destination Zone and Destination Address

Move to the Application tab and make sure the Any box is checked.

Append as needed.

Figure 5.5 - Check the Any box

Head over to the Service/URL Category tab and make sure Any is selected on the URL Category side.

Append as needed.

Figure 5.6 - Check the Any box in URL Category

Head over to the Actions tab, for this example, the Action has been set to Allow. This means the traffic specified in this rule will be allowed to pass. The Log Setting, Log at Session End, has been checked as well.

Append as needed.

Click OK when finished.

Figure 5.7 - Set Action and Log Settings

Commit the changes.

Looking at the logs:

This section can be referred to when trying to find the firewall’s logs

Head over to the Monitor tab and click on Traffic on the left panel.

Clear any filters by clicking on the small X icon next to the filter bar.

Figure 6.1 - Monitor > Traffic

Creating FTP Services (Optional):

Head over to the Objects tab and click on Services on the left panel.

Click on Add on the bottom left.

Figure 7.1 - Objects > Services > Add

A new window will appear called Services.

Set the Service you are creating, in this example it is FTP, so set the Name to service-ftp, set Protocol to TCP, set Destination Port to 20-21 and add Tags as needed.

Append as needed.

Click OK when finished.

Figure 7.2 - Set Name, Protocol, Destination Port, and Tags

Commit the changes.

Adding a NAT policy for the FTP (Optional):

Head back to the Policies tab and click on NAT on the left panel.

Click on Add.

Configure as needed, if following the previous FTP creation follow along.

Set Name to destination-dmz-ftp, set Tags as needed, set Group Rules By Tag as needed, set NAT Type to ipv4, set an Audit Comment if needed.

Figure 8.1 - Set Name, Tags, NAT Type, and Audit Command

Head over to the Original Packet tab in the same window.

For this example, a new Source Zone has been added called inside, the Destination Zone is set to inside, the Destination Interface has been set to ethernet1/2, the Service has been set to service-ftp, the Source Address has been set to Any, and a new Destination Address has been added and checked (192.168.1.1).

Figure 8.2 - Set all parameters as shown

Head over to the Translated Package tab in the window.

For this example, the Translation Type on the right has been set to Static IP and the Translated Address has been set to 192.168.50.10.

Click OK when finished.

Figure 8.4 - Set Translation Type and Translated Address

Commit the changes.

Creating a Security Policy Rule for FTP (Optional):

Head back to the Policies tab and click on Security on the left panel.

Click on Add on the bottom left.

Configure as needed, if following the previous FTP creation follow along.

For this example, set the Name to internal-dmz-ftp, set Rule Type to universal (default), set the Tags and  Group Rules By Tag as needed, set an Audit Comment if needed.

Figure 9.1 - Set Name, Rule Type, Tags, and Audit Comment

Move to the Source tab.

A new Source Zone has been added called inside and the Source Address has been set to Any.

Figure 9.2 - Add Source Zone and set Source Address

Head over to the Destination tab in the same window.

A new Destination Zone has been added called dmz and a new Destination Address has been added (192.168.1.1).

Figure 9.3 - Add Destination Zone and Destination Address

Head over to the Service/URL Category tab in the same window.

A new Service has been added called service-ftp and the Any option has been set for URL Category.

Figure 9.4  - Add Service and set URL Category

Head over to the Actions section in the same window.

Set Action to Allow, on the right side click on the drop-down bar for Schedule and click on New Schedule.

Figure 9.5 - Set Action and Add a new Schedule

A new window will appear called Schedule.

Set Name to ftp-transfer-schedule, set Recurrence to Daily, Add a Start Time of your current time + 5 minutes, and Add an End Time of 2 hours from the Start Time.

Click on OK when finished. Click OK when finished on the previous window as well.

Figure 9.6 - Set Name, Recurrence, Start Time, and End Time

Commit changes made.

REFERENCES:

Network Development Group. (2019). Lab 3: Security and NAT Policies (03). Retrieved from Palo Alto Networks website: https://ve1.netdevgroup.com/local/ex/PAN9FE210_0050_56B5_7BDD_5D48_9788/doc/PAN9_EDU210_Lab_3.pdf